Special Mobility Services

RideSource Vehicle Maintenance Program

Policy:

It is SMS policy to maintain vehicles and equipment in order to provide safe, comfortable, and reliable transportation to our passengers, and effective and efficient service to the community.  

Goals and Objectives:

The goals and objectives of the vehicle maintenance program are:

1.
Maintain vehicles to promote the safety and comfort of passengers, operators, and protect the public.

· Conduct regular pre-trip inspections in order to identify vehicle and equipment problems and assure vehicles are in good operating condition.

· Conduct basic Preventive Maintenance service routines in a timely manner to identify vehicle problems and keep vehicle systems in good repair.

· Conduct vehicle repairs in a timely manner and in accordance with industry best practices. 

· Maintain a clean appearance for vehicles through regular interior and exterior cleaning.  

2.
Manage Preventive Maintenance and repair activities to promote the reliability of the service by minimizing service interruptions due to vehicle or equipment failure.

· Regularly inspect vehicles in order to identify and correct problems in to prevent service interruptions. 

· Schedule repairs promptly in order to minimize service interruptions.

· Utilize subcontractors as needed to perform specialized services and to supplement SMS maintenance staff efforts.

· Schedule SMS PM activities to maximize fleet availability during service peaks.

· Analyze repair, road call and tow data to identify trouble-prone components or systems for pro-active attention.

3.
Maintain vehicles and equipment to promote cost-efficiency of operations.
· Maintain and repair vehicles to ensure their operation at peak efficiency, including fuel efficiency, emissions systems, etc.

· Analyze fleet fuel usage and repair data; identify vehicles which may need remedial work or may need to be made inactive.

· Maintain vehicles and related equipment to fulfill manufacturer’s warranty requirements and pursue warranty repairs where applicable; research and follow up on any applicable recalls or service bulletins.

· Maintain vehicles to maximize the useful vehicle life, including the life of key components such as tires, brakes, batteries, etc.

· Manage the maintenance program to be cost effective in terms of staff time, service vendors and parts and supplies costs.

4.
Conduct vehicle operations, repairs, and cleaning in compliance with applicable local, state and federation regulations.  

· Ensure that shop equipment and maintenance procedures comply with applicable OSHA laws and regulations protecting the health and welfare of workers.

· Handle and dispose of fuels, lubricants, solvents, tires and related materials in a safe and environmentally responsible manner.

· Maintain vehicles to comply with relevant emission standards and other applicable regulations.

· Conduct vehicle cleaning to comply with applicable wastewater and other relevant regulations.

· Conduct maintenance and repairs in compliance with environmental standards and other relevant regulations.

Program Elements:

Pre-trip inspections.  Each vehicle will be inspected at the start of each shift by a driver trained in the procedure.  A walk-around will be performed with a vehicle pre-trip checklist and any irregularities reported to the Mechanic before the vehicle leaves the lot.  Please see Attachments for Pre-Trip Inspection checklist.

Basic Service Routines.  Per the recommendations of the chassis, bus body, and wheelchair lift manufacturers, and the additional recommendations of the SMS Mechanic, a thorough preventive maintenance schedule will be established and followed for each vehicle.  At or before the recommended mileage intervals, the SMS mechanic will perform all the elements of maintenance due at that mileage.  Please see Attachments for Preventive Maintenance Schedules and Standard Operating Procedures.

Vehicle Cleaning.  Interior cleaning and sweeping of each in-service vehicle will be performed at the end of each shift by Special Mobility Services' driving staff.  Vehicle exteriors will be washed on a weekly basis or more frequently, as needed.

Vehicle Repairs.  The need for a vehicle repair may be discovered during a pre-trip inspection, preventive maintenance inspection, or breakdown.  The Mechanic will determine warranty coverage for the system requiring attention, and if appropriate, pursue warranty repairs with the vendor, bus or chassis manufacturer, or authorized warranty outlet.  The SMS Mechanic will determine whether the repair can be accomplished in-house, or because of the need for special diagnostic expertise or equipment, will be assigned to a subcontractor. 

Documentation and Analysis.  Vehicle condition will be regularly documented through pre-trip inspections and problems discovered on the road will be documented on a Vehicle Condition Report by the driver.  In addition, all vehicle maintenance and repair activity and costs will be documented.  Vehicle data will be organized for summary and analysis. 

ATTACHMENTS

	
	Page

	Scheduled maintenance intervals
	

	

Ford E-350
	1

	

Chevrolet Venture
	1

	

Freightliner
	1

	
	

	Inspections and Maintenance:  Active fleet
	

	
Operator
	

	

Pre-trip inspection
	2

	

End-of-shift inspection
	2

	

Daily as-needed
	3

	

Weekly
	3

	
Mechanic
	

	

Daily maintenance
	3

	

Standard (3,000 mile) inspection and service
	4

	

Extended (6,000 mile) inspection and service
	6

	
	

	Inspections and Maintenance:  Contingency fleet
	

	
Mechanic
	

	

Quarterly maintenance
	7

	
	

	Preventive maintenance management
	

	
Mechanic/ Program Manager
	

	

Standard operating procedures
	8

	
	

	Maintenance remediation
	

	
Mechanic/ Program Manager
	

	

Remediation plan
	9

	
	

	Inventory list
	

	
Accessible Services Fleet
	10

	
	

	Sample Vehicle Inspection Forms
	11


SCHEDULED MAINTENANCE INTERVALS

FORD E-450

	Interval
	Associated service

	3,000
	Standard vehicle inspection 1

	
	Engine oil and filter service

	6,000
	Extended vehicle inspection 2

	30,000
	Transmission fluid and filter service

	36,000
	Fuel filter service

	100,000
	Tune up

	
	Spark plug service

	
	Cooling system service

	
	


CHEVROLET VENTURE

	Interval
	Associated service

	5,000
	Standard vehicle inspection 1

	10,000
	Extended vehicle inspection 2

	
	Engine oil and filter service (or as determined by GM Oil Life System)

	25,000
	Fuel filter service

	50,000
	Transaxle fluid and filter service

	100,000
	Tune up

	
	Spark plug service

	150,000
	Cooling system service

	
	


FREIGHTLINER 

	Interval
	Associated service

	5,000
	Standard vehicle inspection 1

	10,000
	Extended vehicle inspection 2

	
	Engine oil and filter change

	
	Fuel filter service

	30,000
	Transmission fluid and filter service

	100,000
	Cooling system service/ replace water temperature regulator

	
	Inspect and adjust valve lash as necessary

	
	


ACTIVE FLEET:  ALL VEHICLES

DAILY PRE-TRIP INSPECTION:  (OPERATOR)

Check exterior:

· Vehicle head, tail, and clearance lights
· Turn signal operation
· Hazard flasher operation
· Brake light operation
· Backup light operation
· Tire condition (check for inflation, tread depth, sidewall damage, objects in tread)

· Wheel condition (check appearance of lug nuts, check for seal leaks)

· Undercarriage leaks
· Body, glass, mirrors (check for damage, mirrors well-secured)
· Wheelchair lift operation (unlock doors, cycle lift once before service)
· Exterior cleanliness
Check vehicle interior:

· Mirror position
· Horn operation.

· Windshield wiper operation
· Passenger door operation
· Interior lights
· Vehicle gauges
· Fuel level (fuel cards and fuel log present)
· Emergency exits (check that exits are not blocked and rear exit door is unlocked)
· Radio/ mobile data operation (pre-trip radio check with dispatcher)
· Seat belts (present and available to passengers)
· Wheelchair securements (all sets present and stowed correctly)
· Safety equipment (fire extinguisher present and fully-charged, emergency triangles present, first aid kit present and sealed or fully-stocked, bio-hazard kit present and sealed or fully-stocked, seat belt cutter present) 

· Seat condition (well-secured, check for upholstery damage)
· Seat belt condition (present, not damaged)
· Stanchions (well-secured)
· Interior cleanliness
DAILY END-OF-SHIFT MAINTENANCE:  (OPERATOR)

· Stow wheelchair securements

· Sweep the coach interior and inspect for damage
· Power off all switches

· Lock vehicle doors

ACTIVE FLEET:  ALL VEHICLES

AS NEEDED MAINTENANCE:  (OPERATOR)

· Fuel as needed
· Wash vehicle exterior as needed (minimum weekly)

· Monitor performance of the climate control systems.
· Clean interior window glass, wipe seats, mop floor

WEEKLY MAINTENANCE:  (OPERATOR)

Daily pre-trip inspection, plus:

· Check engine oil level

· Add oil as needed, log
· Check engine coolant level

· Check condition of belts and hoses
· Fill out weekly vehicle inspection form
DAILY MAINTENANCE:  (MECHANIC)

· Review operator defect reports and repair, schedule for repair, or take vehicle out of service as appropriate
STANDARD VEHICLE INSPECTION 1
Service procedures:

· Lube lift pivot points



Ford only:

· Engine oil change, oil filter change

Freightliner only:

· Lube suspension, steering linkage, and driveline

Items to be inspected:

Vehicle history

· Review preventive maintenance history

· Review vehicle repair history

· Review any pending work orders

Pre-trip inspection

· Gauges

· Switches and controls
· Driver area condition

· Passenger area condition

Drive test

· Starting

· Steering

· Acceleration

· Braking

· Transmission

· Check operation of heating and air conditioning

Under hood

· Listen for and investigate any unusual noises

· Check transmission fluid level and condition

· Check cooling system, coolant level and condition

· Check brake fluid level

· Check power steering fluid level and condition

· Check windshield washer fluid level and condition

· Check fuel lines and connections for leaks
· Inspect and adjust drive belts as needed
· Check coolant hoses for leaks and wear

· Check exposed wiring and vacuum hoses for wear
Freightliner only:

· Clean engine crankcase breather

· Clean and test after-cooler core as needed

· Clean radiator as needed

Under-carriage

· Check tire condition and inflation

· Check wheel condition and lug nut torque
· Check steering linkage for wear

· Check front suspension for looseness or wear

· Check rear suspension for looseness or wear

· Check wheel bearings and seals, front and rear, for play, leakage
· Check brake caliper and rotor condition

· Check brake pad condition

· Check parking brake unit condition

· Check differential for leaks
· Check driveline and U-joint condition

· Check transmission for leakage

· Check exhaust system for leaks or damage

Wheelchair lift

· Cycle lift, inspect and listen for noises

If any defects are found, document and as appropriate, schedule for repair.

EXTENDED VEHICLE INSPECTION 2
Service procedures:

· Brake and wheel bearing service as needed.
· Lube lift pivot points

Ford, Chevrolet only:

· Engine oil change, oil filter change

· Lube chassis and suspension
Freightliner only:

· Engine oil change, oil filter change

· Lube suspension, steering linkage, and driveline

· Replace fuel filters

· Obtain engine oil sample for testing 

· Obtain coolant sample for testing

Standard inspection plus:

Items to be inspected:

Vehicle history

· Incorporate any currently due preventive maintenance items into the current service

Under hood

· Check charging system output

· Check battery and battery cable condition

Under-carriage

· Inspect brakes and wheels

Wheelchair lift

· Check hydraulic hoses and connectors for leaks
· Check bushings for play
If any defects are found, document and as appropriate, schedule for repair.

CONTINGENCY FLEET:  ALL VEHICLES

QUARTERLY MAINTENANCE : 0MECHANIC)

Items to be inspected:

· Check engine fluids

· Check tires for inflation and wear

· Check for fluid leaks under the vehicle

· Start and warm the vehicle to operating temperature

· Run heaters and air conditioners

· Drive the vehicle through the lot, using both forward and reverse

· Check transmission fluid level

If any defects are found, document and as appropriate, schedule for repair.


Accessible Services Fleet

LTD, thru a contract with SMS Services, maintains a total of 34 accessible services vehicles otherwise known as “Ride Source.” The following page represents a breakdown of the various manufacturers and sub fleet information.  

Bus No.  YR.




Manu
   



Model








  VIN 

	22
	1997
	Ford / WorldTrans
	Diplomat Small Bus
	1FDLE40S6VHB13995

	25
	1995
	Ford / WorldTrans
	Diplomat Small Bus
	1FDKE30G3SHB53674

	27
	2001
	Chev Activan
	ElDorado Nat'l Activan
	1GNDX03E31D299148

	31
	1998
	Ford
	WorldTrans Diplomat Small Bus
	1FDXE40S5WHB01341

	36
	2001
	Chev Activan
	ElDorado Nat'l Activan
	1GNDX03E61D298947

	37
	2001
	Ford E-450 Chassis
	ElDorado Nat'l AeroTech 240
	1FDXE45S11HB21894

	38
	2000
	Ford E-450 Chassis
	ElDorado Nat'l AeroTech 240
	1FDXE45S31HB21895

	39
	2000
	Ford E-450 Chassis
	ElDorado Nat'l AeroTech 240
	1FDXE45S71HB21897

	61
	2000
	Ford
	Starcraft Allstar Bus
	1FDXE45S8YHB43093

	62
	2000
	Ford
	Starcraft Allstar Bus
	1FDXE45SXYHB43094

	63
	2000
	Ford
	Starcraft Allstar Bus
	1FDXE45SXYHB65239

	64
	2001
	Ford
	Starcraft Allstar Bus
	1FDXE45S5YHB69103

	65
	2001
	Ford
	Starcraft Allstar Bus
	1FDXE45S4YHB74163

	66
	2001
	Ford E-450 Chassis
	ElDorado Nat'l AeroTech 240
	1FDXE45SX1HB21893

	67
	2001
	Ford E-450 Chassis
	ElDorado Nat'l AeroTech 240
	1FDXE45S51HB21896

	68
	2003
	Ford E-450 Chassis
	ElDorado Nat'l AeroTech 240
	1FDXE45S91HB21898

	69
	2003
	Ford E-450 Chassis
	ElDorado Nat'l AeroTech 240
	1FDXE45S81HB21892

	90
	2003
	Ford S.D. Cutaway
	ElDorado Nat'l AeroTech 240
	1FDXE45S03HB10727

	91
	2003
	Ford S.D. Cutaway
	ElDorado Nat'l AeroTech 240
	1FDXE45S23HB10728

	92
	2003
	Ford S.D. Cutaway
	ElDorado Nat'l AeroTech 240
	1FDXE45S43HB10729

	93
	2003
	Ford S.D. Cutaway
	ElDorado Nat'l AeroTech 240
	1FDXE45S03HB10730

	94
	2003
	Ford S.D. Cutaway
	ElDorado Nat'l AeroTech 240
	1FDXE45S23HB10731

	95
	2005
	Ford S.D. Cutaway
	ElDorado Nat'l AeroTech 240
	1FDXE45S43HB10732

	97
	2005
	Chev Venture
	ElDorado Nat'l Activan
	1GBDX23E63D315242

	110
	2005
	Ford
	ElDorado Aerotech 240
	1FDXE45S05HA77280

	111
	2005
	Ford
	ElDorado Aerotech 240
	1FDXE45S25HA77281

	112
	2005
	Ford
	ElDorado Aerotech 240
	1FDXE45S45HA77282

	113
	2005
	Ford
	ElDorado Aerotech 240
	1FDXE45S65HA77283

	114
	2005
	Ford
	ElDorado Aerotech 240
	1FDXE45S85HA77284

	115
	
	Ford
	ElDorado Aerotech 240
	1FDXE45SX5HA77285

	116
	
	Ford
	ElDorado Aerotech 240
	1FDXE45S15HA77286

	117
	
	Ford
	ElDorado Aerotech 240
	1FDXE45S35HA77287

	118
	
	Chevy Venture chassis
	ElDorado Nat'l Amerivan
	1GBDV13E75D148991

	119
	
	Chevy Venture chassis
	Eldorado Nat'l Amerivan
	1GBDV13E95D150709


PREVENTIVE MAINTENANCE MANAGEMENT:

STANDARD OPERATING PROCEDURES

These procedures apply to the fleet maintenance program at RideSource:
1. At the start of each driving shift, each driver performs a pre-trip inspection to ensure safety and accessibility items are operational, and that any defects are reported to the maintenance staff.

2. Each driver records beginning and ending mileage for the route on that day’s manifest.

3. Each day, dispatch staff record vehicle miles driven for that day to the operations database, and once a week record ending odometer reading for each vehicle.

4. Each week, maintenance staff print a fleet maintenance status report from the operations database, and use projected service due dates from the report to schedule services for each vehicle on or before the service due mileage.  Work is scheduled in-house or out-sourced as needed to meet the maintenance deadlines.

5. Specific components of each vehicle are scheduled for inspection, lubrication, cleaning or replacement at regular intervals.  The intervals are determined by published information from the vehicle and component manufacturers.  In addition, such inspections may include other items or incorporate shorter intervals as recommended by maintenance staff or management.  

6. At each service, maintenance staff record service date, odometer reading, service items, parts used, parts cost, and labor hours to the service checklist, then before end of shift, to the operations database.  

7. For out-sourced preventive maintenance, vendors are supplied with a maintenance checklist.  At each service, maintenance staff obtain copies of the completed checklist and vendor invoice, then enter the service detail to the operations database, identifying the vendor.

8. At regular intervals, management staff audit the data collection process and verify the completeness and timeliness of the database records.  Management also works with maintenance staff to revise maintenance policy and checklists as needed, and to upgrade database capabilities.

9. Monthly, management staff review preventive maintenance detail to ensure timely performance of preventive maintenance services for the prior month, and consult with maintenance staff to ensure adequate resources are available for the workload.

MAINTENANCE REMEDIATION PLAN

The remediation plan calls for the following to be completed by October 31, 2005:

1. Management review of the most recent preventive maintenance procedures for each vehicle in the fleet and identification of any vehicles due for service. 
2. Consultation with the mechanic to ensure he has the time and resources to ensure any maintenance procedures thus identified are completed in a timely manner.

3. Centralization of the vehicle records for both the Eugene and Oakridge operations to the Eugene facility.  

4. Centralization of preventive maintenance scheduling for both program to Eugene.
5. Management review of the vehicle maintenance files and database records for completeness and timeliness.

6. Modifications in data and reports from the fleet management database system to improve preventive maintenance scheduling. 

7. Management review of the preventive maintenance workload, with adjustments made as necessary to ensure the program continues to be adequately staffed and supplied.

SAMPLE VEHICLE INSPECTION FORMS

	Vehicle #
	
	 Employee

	
	
	

	 Odometer
	
	 Date


	
	 
	Pre-inspection
	 

	
	 
	 Print, review, and attach a copy of service detail
	 

	
	 
	 Review any current write-ups for this vehicle
	 


	
	Odometer
	
	Date
	
	Most recent

	
	
	
	
	
	Oil change


	Inspect
	Repair
	
	

	OK
	Needed
	
	Comments


	 
	 
	Pre-trip inspection
	 

	 
	 
	 Gauges
	 

	
	
	 Switches and controls
	

	
	
	 Driver area condition
	

	
	
	 Passenger area condition
	


	 
	 
	Road test
	 

	 
	 
	 Starting
	 

	
	
	 Steering
	

	
	
	 Acceleration
	

	
	
	 Braking
	

	
	
	 Transmission
	

	
	
	 Heating and air conditioning
	


	 
	 
	Engine compartment: engine running
	 

	
	
	 Listen for and investigate any unusual noises
	

	 
	 
	 Transmission fluid level and condition
	 


	 
	 
	Engine compartment: engine off
	 

	 
	 
	 Cooling system, coolant level and condition
	 

	
	
	 Brake fluid level
	

	 
	 
	 Power steering fluid level and condition
	 

	 
	 
	 Windshield washer fluid level and condition
	 

	 
	 
	 Fuel lines and connections
	 

	 
	 
	 Inspect and adjust drive belts as needed
	 

	 
	 
	 Coolant hoses for leaks and wear
	 

	 
	 
	 Exposed wiring and vacuum hoses for wear
	 

	
	
	 Freightliner only:
	

	
	
	 Clean engine crankcase breather
	

	
	
	 Clean and test after-cooler core as needed
	

	
	
	 Clean radiator as needed
	


	 
	 
	Undercarriage
	 

	
	
	 Tire condition and inflation
	

	
	
	 Wheel condition and lug nut torque
	

	
	
	 Steering linkage for wear
	

	 
	 
	 Front suspension for looseness or wear
	 

	 
	 
	 Rear suspension for looseness or wear
	 

	 
	 
	 Wheel bearings and seals, front and rear
	 

	
	
	 Brake caliper and rotor condition
	

	
	
	 Brake pad condition
	

	 
	 
	 Parking brake unit condition
	 

	 
	 
	 Differential for leaks
	 

	 
	 
	 Driveline and U-joint condition
	 

	 
	 
	 Transmission for leakage
	 

	 
	 
	 Exhaust system for leaks or damage
	 

	
	
	 Ford only:
	

	
	
	 Change engine oil and oil filter
	

	
	
	 Freightliner only:
	

	
	
	 Lube suspension, steering, and driveline
	


	 
	 
	Wheelchair lift
	 

	
	
	 Cycle lift, inspect, and listen for noises
	

	
	
	 Lubricate lift pivot points
	


	 
	 
	Other scheduled maintenance
	 

	
	
	
	

	
	
	
	


	 
	 
	Other unscheduled maintenance
	 

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	


	Vehicle #
	
	 Employee

	
	
	

	 Odometer
	
	 Date


	
	 
	Pre-inspection
	 

	
	 
	 Print, review, and attach a copy of service detail
	 

	
	 
	 Review any current write-ups for this vehicle
	 


	
	Odometer
	
	Date
	
	Most recent

	
	
	
	
	
	Oil change

	
	
	
	
	
	Transmission service

	
	
	
	
	
	Fuel filter service

	
	
	
	
	
	Brake service

	
	
	
	
	
	Tune up/ spark plug service

	
	
	
	
	
	Coolant system service


	Inspect
	Repair
	
	

	OK
	Needed
	
	Comments


	 
	 
	Pre-trip inspection
	 

	 
	 
	 Gauges
	 

	
	
	 Switches and controls
	

	
	
	 Driver area condition
	

	
	
	 Passenger area condition
	


	 
	 
	Road test
	 

	 
	 
	 Starting
	 

	
	
	 Steering
	

	
	
	 Acceleration
	

	
	
	 Braking
	

	
	
	 Transmission
	

	
	
	 Heating and air conditioning
	


	 
	 
	Engine compartment: engine running
	 

	
	
	 Listen for and investigate any unusual noises
	

	 
	 
	 Transmission fluid level and condition
	 

	
	
	 Charging system output
	


	 
	 
	Engine compartment: engine off
	 

	 
	 
	 Cooling system, coolant level and condition
	 

	
	
	 Brake fluid level
	

	 
	 
	 Power steering fluid level and condition
	 

	 
	 
	 Windshield washer fluid level and condition
	 

	 
	 
	 Fuel lines and connections
	 

	 
	 
	 Inspect and adjust drive belts as needed
	 

	 
	 
	 Coolant hoses for leaks and wear
	 

	 
	 
	 Exposed wiring and vacuum hoses for wear
	 

	
	
	 Battery and battery cable condition
	

	
	
	 Freightliner only:
	

	
	
	 Replace fuel filters
	

	
	
	 Clean engine crankcase breather
	

	
	
	 Clean and test after-cooler core as needed
	

	
	
	 Clean radiator as needed
	


	 
	 
	Undercarriage
	 

	
	
	 Tire condition and inflation
	

	
	
	 Wheel condition and lug nut torque
	

	
	
	 Steering linkage for wear
	

	 
	 
	 Front suspension for looseness or wear
	 

	 
	 
	 Rear suspension for looseness or wear
	 

	 
	 
	 Wheel bearings and seals, front and rear
	 

	
	
	 Brake caliper and rotor condition
	

	
	
	 Brake pad condition
	

	 
	 
	 Parking brake unit condition
	 

	 
	 
	 Differential for leaks
	 

	 
	 
	 Driveline and U-joint condition
	 

	 
	 
	 Transmission for leakage
	 

	 
	 
	 Exhaust system for leaks or damage
	 

	
	
	 Ford, Chevrolet only:
	

	
	
	 Change engine oil and oil filter
	

	
	
	 Lube suspension and chassis
	

	
	
	 Service brakes and wheel bearings per inspection
	

	
	
	 Freightliner only:
	

	
	
	 Change engine oil and oil filter
	

	
	
	 Lube suspension, steering linkage, and driveline
	

	
	
	 Service brakes and wheel bearings per inspection
	

	
	
	 Obtain engine oil sample for testing
	

	
	
	 Obtain coolant sample for testing
	


	 
	 
	Wheelchair lift
	 

	 
	 
	 Cycle lift, inspect and listen for noises
	 

	
	
	 Lubricate pivot points
	

	
	
	 Hydraulic hoses and connectors for leaks
	

	
	
	 Bushings for play
	


	 
	 
	Other scheduled maintenance
	 

	
	
	
	

	
	
	
	


	 
	 
	Other unscheduled maintenance
	 

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	


RideSource Preventive Maintenance Program


